Pin Usage

December 10, 2022

11:00 AM

| GPIO | Input | Output | Function | ADC | DAC | RTC | Capacitive Touch | WT32-SC01 | CADDeck Std | CADDeck 10 Buttons | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | pulled up | OK |  | ADC2_CH1 |  | RTC_GPI011 | T1 | I2S_MCLK (Available) |  |  | outputs PWM signal at boot, must be LOW to enter flashing mode |
| 1 | TX pin | OK |  |  |  |  |  |  |  |  | debug output at boot |
| 2 | OK | OK |  | ADC2_CH2 |  | RTC_GPI012 | T2 | Available |  |  | connected to on-board LED, must be left floating or LOW to enter flashing mode |
| 3 | OK | RX pin |  |  |  |  |  |  |  |  | HIGH at boot |
| 4 | OK | OK |  | ADC2_CH0 |  | RTC_GPI010 | T0 |  |  |  |  |
| 5 | OK | OK |  |  |  |  |  | I2S_SCLK (Available) |  |  | outputs PWM signal at boot, strapping pin. I2S is a digital audio interface. |
| 6 | x | x | (SCK/CLK) |  |  |  |  |  |  |  | connected to the integrated SPI flash |
| 7 | x | x | (SDO/SD0) |  |  |  |  |  |  |  | connected to the integrated SPI flash |
| 8 | x | x | (SDI/SD1) |  |  |  |  |  |  |  | connected to the integrated SPI flash |
| 9 | x | x | (SHD/SD2) |  |  |  |  |  |  |  | connected to the integrated SPI flash |
| 10 | x | x | (SWP/SD3) |  |  |  |  |  |  |  | connected to the integrated SPI flash |
| 11 | x | x | (CSC/CMD) |  |  |  |  |  |  |  | connected to the integrated SPI flash |
| 12 | OK | OK |  | ADC2_CH5 |  | RTC_GPI015 | T5 | (TFT_MISO) |  |  | boot fails if pulled high, strapping pin |
| 13 | OK | OK |  | ADC2_CH4 |  | RTC_GPI014 | T4 | LCD_SDA (TFT_MOSI) |  |  |  |
| 14 | OK | OK |  | ADC2_CH6 |  | RTC_GPI016 | T6 | Output LCD_SCL (TFT_SCLK) |  |  | outputs PWM signal at boot |
| 15 | OK | OK |  | ADC2_CH3 |  | RTC_GPI013 | T3 | TFT_CS |  |  | outputs PWM signal at boot, strapping pin |
| 16 | OK | OK |  |  |  |  |  | Not available on WROVER-B |  |  |  |
| 17 | OK | OK |  |  |  |  |  | Not available on WROVER-B |  |  |  |
| 18 | OK | OK |  |  |  |  |  | I2C_SDA (TP,EM20918) | Also PC8575 SDA | Also PC8575 SDA | Pulled to 3.3V through 10k |
| 19 | OK | OK |  |  |  |  |  | I2C_SCL (TP,EM20918) | Also PC8575 SCL | Also PC8575 SCL | Pulled to 3.3V through 10k |
| 21 | OK | OK |  |  |  |  |  | Output LCD_RS |  |  |  |
| 22 | OK | OK |  |  |  |  |  | Input LCD_REST |  |  |  |
| 23 | OK | OK |  |  |  |  |  | Output - LCD_LED (Display brightness) |  |  |  |
| 25 | OK | OK |  | ADC2_CH8 | DAC1 | RTC_GPI06 |  | I2S_LRCK (Available) | ENC_1_A |  | I2S is a digital audio interface. |
| 26 | OK | OK |  | ADC2_CH9 | DAC2 | RTC_GPI07 |  | I2S_DSDIN (Available) | ENC_1_B |  | I2S is a digital audio interface. |
| 27 | OK | OK |  | ADC2_CH7 |  | RTC_GPI017 | T7 | Available | PCF8575 INT | PCF8575 INT |  |
| 32 | OK | OK |  | ADC1_CH4 |  | RTC_GPI09 | T9 | Available | Roll (X) | Roll (X) |  |
| 33 | OK | OK |  | ADC1_CH5 |  | RTC_GPI08 | T8 | Available | Pitch (Y) | Pitch (Y) |  |
| 34 | OK | NO |  | ADC1_CH6 |  | RTC_GPI04 |  | Available | ENC_2_A (future) | Zoom (analog) | No pullup resistor available |
| 35 | OK | NO |  | ADC1_CH7 |  | RTC_GPI05 |  | I2S_ASDOUT (Available) | ENC_2_B (future) | Rotate (Analog) | No pullup resistor available |
| 36 | OK | NO |  | ADC1_CH0 |  | RTC_GPI00 |  | SENSOR_VP, EM20918 EM_INT |  |  | No pullup resistor available |
| 37 |  |  |  | ADC1_CH1 |  |  |  | Not available |  |  | Not Available on WROVER-B |
| 38 |  |  |  | ADC1_CH2 |  |  |  | Not available |  |  | Not available on WROVER-B |
| 39 | OK | NO |  | ADC1_CH3 |  | RTC_GPI03 |  | Touch Interrupt, SENSOR_VN |  |  | No pullup resistor available |

PCF8575 Pinout (10 Buttons) and Push-Sensor

| Pin | Function |  | Pin | Function |
| --- | --- | --- | --- | --- |
| P0 | PUSH_SENSOR_KNOB |  | P8 |  |
| P1 | BUTTON_1_R |  | P9 |  |
| P2 | BUTTON_2_R |  | P10 |  |
| P3 | BUTTON_3_R |  | P11 | BUTTON_1_L |
| P4 | BUTTON_4_R |  | P12 | BUTTON_2_L |
| P5 | BUTTON_5_R |  | P13 | BUTTON_3_L |
| P6 |  |  | P14 | BUTTON_4_L |
| P7 |  |  | P15 | BUTTON_5_L |

Move to bits 0-7, and/or use a PCF8574 in any future version

PCF8575 Pinout (Std)

| Pin | Function |  | Pin | Function |
| --- | --- | --- | --- | --- |
| P0 |  |  | P8 | BUTTON_1 |
| P1 |  |  | P9 | BUTTON_2 |
| P2 |  |  | P10 | BUTTON_3 |
| P3 |  |  | P11 | BUTTON_4 |
| P4 |  |  | P12 | BUTTON_JOY_1 |
| P5 |  |  | P13 | BUTTON_JOY_2 |
| P6 |  |  | P14 | ENC_1_SW |
| P7 |  |  | P15 | ENC_2_SW |

Move to bits 0-7, and/or use a PCF8574 in any future version